Improving the Inverse Square Law Experiment When Using the

White LED Light Source Assembly and the Light Sensor and Holder

Richard G. Born

Northern Illinois University

Operations Management and Information Systems

The Vernier white LED light source assembly shows a significant decrease in illumination with time as the light source assembly warms up:

During the time that it might take for a typical student to perform the inverse square law experiment, say 15 to 20 minutes actually collecting data, this reduction in illumination over time would be expected to increase the value of the exponent obtained for the inverse “square” law:

The blue line on the above Excel chart shows the results when the experiment is performed by starting with the lamp at room temperature.  The red line shows the results when the experiment is performed by starting with the lamp warmed up for about an hour.  An hour was selected since the illumination of the LED decreases relatively slowly after an hour of warm up time.  Without LED warm-up the inverse “square” law provided a power of 2.236.  With an hour of warm-up time the inverse “square” law provided a power of 1.965 – much closer to the desired value of 2.

Clearly, students should be advised of the need to have the Vernier white source LED properly warmed up in order to obtain the best possible inverse square law results.

Attachments:

3 LoggerPro Files (with original data from a Labquest)

1 Excel File (from which the above two charts were constructed)

1 Word File (this document)