Visible Spectra of Commercial Dyes:
the Forensic Version

A star athlete at your school became ill during a critical tournament contest. It was reported that the athlete suffered from “flu-like” symptoms, and was treated and released after spending the night in a local hospital.

Laboratory test results from the hospital indicated that the athlete had ingested a sufficiently large amount of copper (II) sulfate to cause severe gastrointestinal distress. When interviewed by the local police, the athlete said that one of the bottles of sports drink in the locker room tasted “slightly off”, but thought nothing of it until being stricken with nausea and cramps.

Investigators from your laboratory gathered all the different sports drinks in the locker room. It will be your job to use a spectrometer to test each sports drink and determine which one had been laced with copper (II) sulfate. The problem is complicated by the facts that copper (II) sulfate imparts a blue color to water when it is dissolved, and many of the sports drinks contain blue food dyes with colors that look very similar to copper (II) sulfate.

In this experiment, you will use a Vernier SpectroVis Plus spectrometer to identify the dyes in the sports drinks. To help your investigation, you will have samples of all of the FD&C food dyes approved for use in liquid food products, as well as a sample of copper (II) sulfate.

In Part I of this experiment, you will measure the absorbance spectrum of each of the FD&C food dyes and the CuSO4 solution so that you have a set of standards against which to compare the sports drinks. In Part II, you will measure the absorbance spectrum of each of the sports drinks found in the locker room. By comparing the standards with the drinks you should be able to identify the sports drink that had been “poisoned” with CuSO4.

OBJECTIVES

In this experiment, you will

- Measure and analyze the visible light absorbance spectra of samples of aqueous food dye mixtures and a CuSO4 solution to determine the absorbance spectrum for each sample.

- Use the absorbance spectra graphs of food dyes and CuSO4 solution to determine the composition of the coloring agents in samples of sports drinks.

- Determine which sports drink contains CuSO4 solution.

MATERIALS

| Vernier SpectroVis Plus | FD&C food dye solutions |
| --- | --- |
| computer | 0.1 M CuSO4 solution |
| Vernier Logger Pro 3 software | commercial sports drinks |
| one cuvette | unknown sports drink |
| 100 mL graduated cylinder | distilled water |
| 250 mL beakers | tissues (preferably lint-free) |
| plastic Beral pipets | stirring rod |

PROCEDURE

PART I  Measure the Absorbance Spectra of the Standard Solutions

1.	Obtain and wear goggles.

2.	Use a USB cable to connect a Vernier SpectroVis spectrometer to your computer.

3.	Start the Logger Pro 3 (version 3.8.5 or newer) program on your computer.

4.	Obtain a sample of the FD&C dye Red 40.

5.	Calibrate the spectrometer.

- Prepare a blank by filling an empty cuvette ¾ full with distilled water.

- Open the Experiment menu and select Calibrate ► (Spectrometer). The following message appears in the Calibrate dialog box: “Waiting 90 seconds for the device to warm up.” After 90 seconds, the message changes to: “Warmup complete.”

- Place the blank in the spectrometer. Align the cuvette so that the clear sides are facing the light source of the spectrometer. Click “Finish Calibration”, and then click .

6.	Conduct a full spectrum analysis of a food coloring sample.

- Empty the blank cuvette and rinse it twice with small amounts of a food coloring mixture. Fill the cuvette ¾ full with the food coloring mixture and place it in the spectrometer. Align the cuvette so that the clear sides are facing the light source of the spectrometer.

- Click . A full spectrum graph of the food coloring sample will be displayed.

- Click . Examine the graph, noting the peak or peaks of very high absorbance or other distinguishing features.

7.	To save your graph, select Store Latest Run from the Experiment menu.

A LITTLE BIT ABOUT HOW SPECTROSCOPY WORKS

Your food dye sample will look similar to the graph below. The spectrometer projects light toward the cuvette of Red 40 solution and the molecules in the solution allow some of the wavelengths of light to pass through the sample and reach the spectrometer’s detector. The graph shows you that, in order for your eyes to see a red liquid, certain wavelengths (colors) of light are absorbed and others are not. Note the colors (wavelengths) where the peaks appear on the graph. As you test the other food dyes, it will be clear why these liquids display their particular colors. It will also be interesting to see the differences between two dyes of the same “color”.

FD&C Red 40 Absorbance Spectrum

There is a great deal more to spectroscopy. Your instructor may guide you to read more about this method of analysis.

8.	Repeat Steps 6 and 7 with the remaining food coloring samples and the CuSO4 solution.

Note 1: To rename the runs, double click on the title of a run in the Table. Type in a new name (such as “FD&C Blue 40”) to better identify your set of data.

Note 2: To manage the plots you will see on the graph, double click anywhere on the graph itself. In the Graph Options dialog box, click the tab called “Axes Options”. A list of your runs will appear under “Y-axis Columns”. Check the boxes of the run or runs that you want to see on the graph.

PART II Measure the Absorbance Spectra of the Sports Drink Samples

9.	Obtain samples of the sports drinks found in the locker room. Repeat Steps 6 and 7 with the sports drinks.

10.	To save your data in a file, open the File menu and choose Save As… Give your file a memorable name and save it in a place where you can easily retrieve it to complete your data analysis and prepare your report.

DATA ANALYSIS

1.	Describe, in detail, the absorbance spectrum of each food dye sample and the CuSO4 solution. Emphasize the distinguishing features of each spectrum. Include the wavelengths and absorbance values of every peak in the graph of each food dye.

2.	Identify the food dye or dyes present in the sports drinks that you tested. Support your identification with specific information from your testing.

3.	Identify the specific bottle of sports drink that was “poisoned” with the CuSO4 solution.