Oxygen and Aerobic 
Metabolism

Oxygen plays a key role in aerobic cellular metabolism, facilitating the conversion of glucose, protein, and lipids into usable energy. For every 6 molecules of oxygen used in the breakdown of glucose, 6 molecules of carbon dioxide are produced, along with water and adenosine triphosphate (ATP), according to the following equation:

C6H12O6 + 6O2 → 6CO2 + 6 H2O + energy (heat or ATP)

The average person uses 200–250 mL of O2 per minute at rest. This may increase to 2–3 L per minute during heavy exercise and to twice that amount in highly trained athletes. The increase of oxygen consumption is proportional to the amount of work performed up to a maximum level which is dependent on conditioning. At the start of exercise, anaerobic metabolism is used briefly, but this quickly changes to aerobic metabolism as blood flow to muscles increases.

As O2 is consumed and CO2 is produced by muscle cells (and other cells), a pressure gradient is created between the cells, the interstitial fluid, and the bloodstream. A marked lowering of O2 in interstitial fluid as it is used up by cells leads to O2 diffusion from the bloodstream. While dissolved O2 provides an immediate supply to replenish the interstitial fluid and cells, the majority of O2 is carried on hemoglobin molecules. Oxyhemoglobin dissociates more readily as the oxygen concentration is lowered (and CO2 concentration increased), rapidly replenishing the supply of dissolved O2. CO2 diffuses from active cells (where it is produced in high concentration) to the interstitial fluid and bloodstream (where it is transported to the lungs mainly as bicarbonate). In the lungs, the opposite is true. O2 follows a pressure gradient from the alveoli into the bloodstream, and CO2 from the bloodstream into the alveoli.

In this experiment, you will measure tidal volumes and oxygen concentrations of deeply exhaled air at rest and after exercise. You will use these measurements to calculate the resulting differences in oxygen consumption.

Important: Do not attempt this experiment if you have pulmonary or musculoskeletal problems that might be aggravated by exercise.

Objectives

In this experiment, you will

- Obtain graphical representation of exhaled volumes and changes in O2 concentration with breathing at rest and after exercise.

- Calculate oxygen consumption at rest and after exercise.

- Correlate your findings with clinical situations.

MATERIALS

| computer | Vernier O2-Spirometer adapter |
| --- | --- |
| Vernier computer interface | disposable bacterial filter |
| Logger Pro | nose clip |
| Vernier Spirometer |  |
| Vernier O2 Gas Sensor |  |

PROCEDURE

Part I  Oxygen utilization at rest

1.	Assemble the apparatus (see Figure 1).

- Insert the O2 Gas Sensor into the vertical opening of the O2-Spirometer adapter.

- Identify the side of the Spirometer head marked “Inlet” and insert it into the adapter (the larger diameter side).

Attach the bacterial filter to the smaller end of the adapter as shown. Caution: Do not puncture the filter material inside the bacterial filter.

Figure 1

2.	Connect the Spirometer and the O2 Gas Sensor to the Vernier computer interface. Open the file “Aerobic Metabolism.”

3.	Hold the apparatus in one or both hands. Brace your arm(s) against a solid surface, such as a table, and click . Check Spirometer in the pop-up box that appears. Do not zero the O2 Gas Sensor. Note: The Spirometer must be held straight up and down during data collection (see Figure 1).

4. 	Verify that the O2 Gas Sensor is reading close to 21%. If the O2 Gas Sensor is not reading close to 21%, perform a standard two-point calibration before proceeding.

5. 	Collect data.

- Put on the nose clip.

- Inhale deeply and hold your breath for a full 5 seconds. Note: Do not inhale through the apparatus during this inhalation.

- After 5 seconds, click  to begin data collection.

- Exhale slowly through the apparatus. Note: The exhalation should last for 1 second or longer.

- Data collection will continue for 60 seconds.

6.	Determine the volume of exhalation.

- Click on the top left-hand corner of the Volume vs. Time graph.

- Choose Statistics from the Analyze Menu.

- Enter the y value to the nearest 0.1 L as Pre-exercise Exhalation Volume in Table 1.

- Minimize or close the Statistics helper window.

7.	Determine the change in oxygen concentration.

- Click on the top left-hand corner of the Oxygen Gas vs. Time graph.

- Choose Statistics from the Analyze Menu.

- Enter the y value to the nearest 0.01 % as Pre-exercise ∆O2 Concentration in Table 1.

- Minimize or close the Statistics helper window.

8.	Choose Store Latest Run from the Experiment menu to store the data.

9.	Disassemble the apparatus. Gently wave the O2 Gas Sensor and Spirometer back and forth to remove residual expired air.

Part II  Oxygen utilization during exercise

10.	Reassemble the apparatus. Verify that the O2 Gas Sensor is again reading close to 21%. If not, allow it to sit until it has returned to a stable reading near 21%.

11. 	Begin running in place for 3 minutes, moving arms as well as legs.

12.	At the end of 3 minutes, pick up the apparatus in one or both hands. Brace your arm(s) against a solid surface, such as a table, and click . Check Spirometer in the pop-up box that appears. Do not zero the O2 Gas Sensor. Note: The Spirometer must be held straight up and down during data collection (see Figure 1).

13.	Collect data.

- Put on the nose clip.

- Inhale deeply and then hold your breath for a full 5 seconds. Note: Do not inhale through the apparatus during this inhalation.

- After 10 seconds, click  to begin data collection.

- Exhale slowly through the apparatus. Note: The exhalation should last for 1 second or longer.

- Data collection will continue for 60 seconds.

14.		Determine the volume of exhalation.

- Click on the top left-hand corner of the Volume vs. Time graph.

- Choose Statistics from the Analyze menu.

- Select Latest | Volume and then select OK.

- Enter the y to the nearest 0.1 L, as Post-exercise Exhalation Volume in Table 1.

15.		Determine the change in oxygen concentration.

- Select Latest | Oxygen Gas and then select OK.

- Click on the top left-hand corner of the Oxygen Gas vs. Time graph.

- Enter the y value displayed in the lower left corner of the graph to the nearest 0.01%, as Post-exercise ∆O2 Concentration in Table 1.

DATA

| Table 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Exhaled Volume (L) |  | ∆O2 Concentration (%) |  |
|  | Pre-exercise | Post-exercise | Pre-exercise | Post-exercise |
| Exhalation |  |  |  |  |

Data Analysis

1.	Use the values for Exhaled Volume and ∆O2 Concentration (%) from Table 1 to calculate the O2 consumed pre- and post-exercise per breath.

∆O2 Concentration (%) ´ Exhaled Volume (L) = Volume of O2 consumed per breath

2.	CO2 was not measured. What would you expect the volume of exhaled CO2 to be in this experiment at rest and after exercise?

EXTENSIONs

1.	Oxygen consumption with exercise is directly proportional to the muscle mass being used. Demonstrate this principle by performing this experiment exercising with your legs only and/or with your arms only.

2.	Perform this experiment after exercising for varying lengths of time (1, 3, 5, and 10 minutes).

3. 	Repeat this experiment 4 times to determine the average volume of oxygen consumed per individual.

4.   Repeat this experiment, but compare men to women or athletes to non-athletes.

5. 	Oxygen consumption is correlated with heart rate during exercise. Use a Heart Rate Monitor and this experiment to determine if increased oxygen consumption is correlated with an increase in heart rate.