Oxygen and Aerobic Metabolism

1.	The following information refers to a modification of Experiment 22 from Human Physiology with Vernier. The original version measured the volume and oxygen gas content of a series of inhalations and exhalations. This version measures the oxygen gas content of a single exhalation, and thus does not require a Spirometer. The student instructions for all versions of this activity can be found at
http://www.vernier.com/innovate/tips-for-measuring-oxygen-consumption-and-vo2-max/

2.	The apparatus should be held steadily with the O2 Gas Sensor positioned vertically. It is helpful to brace at least one arm on a hard surface, such as a table. Deviation from this position may affect data collection.

3.	It is important to make sure that the O2 Gas Sensor is reading close to 21% before each run. If the sensor is not reading close to 21%, have the students calibrate the sensor.

4.	This protocol does not use a Spirometer. As a result, the students use 4 L as an estimate of the pre and post-exercise exhalation volume.

5.	 To maintain good hygiene, clean the BioChamber 250 with soap and water and allow to air dry after use. As this protocol does not include the use of a bacterial filter, students must not share or exchange BioChambers during this experiment.

SAMPLE Data

Percent oxygen gas for exhalations at rest and after exercise

| Table 1 |  |
| --- | --- |
| ∆O2 Concentration (%) |  |
| Pre-exercise | Post-exercise |
| 5.1 | 9.01 |

Answers to the data analysis Questions

1.	Answers will vary. ∆O2 Concentration (%)  Exhaled Volume (L) = O2 consumed per breath.

Pre-exercise ∆O2 Concentration = 5.09%

Assume pre-exercise Exhaled Volume = 4 L

(0.051)(4 L/breath) = 0.204 L/breath

Post-exercise ∆O2 Concentration = 9.01%

Assume post-exercise Exhaled Volume = 4 L

(0.090)(4 L/breath) = 0.360 L/breath

At rest, 0.204 L of O2 were consumed. This is higher than what is found on precision medical equipment. O2 consumption increased to 0.360 L after three minutes of exercise.

2.	The volume of exhaled CO2 should be roughly equivalent to the volume of O2 consumed.