Oxygen and Aerobic Metabolism

1.	The following information refers to a modification of Experiment 22 from Human Physiology with Vernier. The original version measured the volume and oxygen gas content of a series of inhalations and exhalations. This version measures the volume and oxygen gas content of a single exhalation. The student instructions for all versions of this activity can be found at http://www.vernier.com/innovate/tips-for-measuring-oxygen-consumption-and-vo2-max/

2.	The Spirometer should be held vertically and steadily. It is helpful to brace at least one arm on a hard surface, such as a table. Deviation from this position will affect data collection.

3.	It important to make sure that students zero the Spirometer in the proper orientation before each use. If the sensor is not zeroed properly, deviations in exhaled volume will be observed.

4.	It is important to make sure that the O2 Gas Sensor is reading close to 21% before each run. If the sensor is not reading close to 21%, have the students calibrate the senor.

5.	It may be difficult for students to exhale normally and slowly through the apparatus after exercise. This can lead to erroneous measurements of exhaled volume. If the second exhaled volume exceeds 6 L, have the students use 4 L for the post-exercise exhalation volume.

6.	To maintain good hygiene, use the Spirometer with a MicroGard® bacterial filter and a disposable mouthpiece. Every student must have his or her own bacterial filter and disposable mouthpiece.

SAMPLE Data

Volume and percent oxygen for exhalations at rest and after exercise

| Table 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Exhaled Volume (L) |  | ∆O2 Concentration (%) |  |
|  | Pre-exercise | Post-exercise | Pre-exercise | Post-exercise |
| Breath | 3.1 | 4.2 | 5.09 | 9.01 |

Answers to the data analysis Questions

1.	Answers will vary. ∆O2 Concentration (%)  Exhaled Volume (L) = O2 consumed per breath.

Pre-exercise ∆O2 Concentration = 5.09%

Pre-exercise Exhaled Volume = 3.1 L

(0.051)(3.1L/breath) = 0.158 L/breath

Post-exercise ∆O2 Concentration = 9.01%

Post-exercise Exhaled Volume = 4.2 L

(0.090)(4.2L/breath) = 0.378 L/breath

At rest, 0.158 L of O2 were consumed. This is higher than what is found on precision medical equipment. O2 consumption increased to 0.378 L after three minutes of exercise.

2.	The volume of exhaled CO2 should be roughly equivalent to the volume of O2 consumed.